#include <ESP8266WiFi.h>

#include <ESP8266WebServer.h>

#include <Wire.h>

#include <Servo.h>

#include "MAX30100_PulseOximeter.h"

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

// ======= USER CONFIG =======

const char* SSID = "YOUR_SSID";

const char* PASSWORD = "YOUR_PASSWORD";

const uint16_t REPORTING_PERIOD_MS = 1000; // OLED / Serial / web data update interval

// Thresholds (tweak as required)

const float BPM_THRESHOLD = 100.0;   // example threshold for heart rate

const float TEMP_THRESHOLD = 38.0;   // example threshold in °C

// Pins

const int SERVO_PIN = D4; // GPIO2 - servo signal

// LM35 -> A0 (ADC)

// MAX30100 uses I2C (SDA D2, SCL D1)

const int LM35_PIN = A0;

// ======= HARDWARE OBJECTS =======

ESP8266WebServer server(80);

PulseOximeter pox;

Adafruit_SSD1306 display(128, 64, &Wire, -1);

Servo myServo;

// ======= GLOBAL STATE =======

volatile float currentBPM = 0.0;

volatile float currentSpO2 = 0.0;

volatile float currentTemp = 0.0;

bool hasTurned = false;      // ensure servo turns only once

bool waiting = false;        // waiting during 10s non-blocking delay

unsigned long waitStart = 0;

const unsigned long WAIT_DURATION = 10000UL; // 10 seconds

unsigned long tsLastReport = 0;

// ======= HTML page (ptr) - minimal JS polls /data every second =======

String ptr = "";

ptr += "<!DOCTYPE html>";

ptr += "<html>";

ptr += "<head>";

ptr += "  <meta name='viewport' content='width=device-width, initial-scale=1'>";

ptr += "  <title>Patient Health Monitor</title>";

ptr += "  <link rel='stylesheet' href='https://cdnjs.cloudflare.com/ajax/libs/font-awesome/5.7.2/css/all.min.css'>";

ptr += "  <style>";

ptr += "    body{font-family:Arial,Helvetica,sans-serif;background:#f5f8fa;margin:0;padding:0;display:flex;justify-content:center;align-items:center;height:100vh}";

ptr += "    .card{background:#fff;padding:18px;border-radius:10px;box-shadow:0 6px 18px rgba(0,0,0,0.08);width:320px}";

ptr += "    h1{font-size:20px;color:#2c3e50;margin:0 0 12px 0;text-align:center}";

ptr += "    .row{display:flex;justify-content:space-between;padding:8px 0;border-bottom:1px solid #eee}";

ptr += "    .label{color:#666}";

ptr += "    .value{font-weight:700}";

ptr += "    .heart{color:#ff3b30}";

ptr += "    .spo2{color:#007bff}";

ptr += "    .temp{color:#ff9900}";

ptr += "  </style>";

ptr += "  <script>";

ptr += "    function fetchData(){";

ptr += "      fetch('/data').then(r=>r.json()).then(d=>{";

ptr += "        document.getElementById('bpm').innerText = d.bpm;";

ptr += "        document.getElementById('spo2').innerText = d.spo2;";

ptr += "        document.getElementById('temp').innerText = d.temp;";

ptr += "      }).catch(e=>console.log('err',e));";

ptr += "    }";

ptr += "    setInterval(fetchData,1000);";

ptr += "    window.onload = fetchData;";

ptr += "  </script>";

ptr += "</head>";

ptr += "<body>";

ptr += "  <div class='card'>";

ptr += "    <h1>Patient Health Monitor</h1>";

ptr += "    <div class='row'><div class='label'>Heart Rate</div><div class='value heart' id='bpm'>--</div></div>";

ptr += "    <div class='row'><div class='label'>SpO2</div><div class='value spo2' id='spo2'>--</div></div>";

ptr += "    <div class='row'><div class='label'>Temperature (°C)</div><div class='value temp' id='temp'>--</div></div>";

ptr += "  </div>";

ptr += "</body>";

ptr += "</html>";

// ======= Callbacks =======

void onBeatDetected() {

// optional: you can flash an LED here or log

}

// ======= Helper functions =======

float readLM35Celsius() {

// Convert ADC reading to voltage and then to Celsius for LM35 (10mV/°C).

// On NodeMCU ADC range is 0-1.0V mapped to 0-1023 (if using default core),

// but many NodeMCU dev boards provide ADC scaled to 0-3.3V.

// We will assume ADC range is 0 - 3.3V here. If your board differs, change VREF.

const float VREF = 3.3; // adjust if your ADC reference differs

int adc = analogRead(LM35_PIN);

float voltage = adc * (VREF / 1023.0);

float tempC = voltage * 100.0; // LM35: 10mV per °C

return tempC;

}

String jsonData() {

String s = "{";

s += "\"bpm\":"; s += String(currentBPM, 1); s += ",";

s += "\"spo2\":"; s += String(currentSpO2, 1); s += ",";

s += "\"temp\":"; s += String(currentTemp, 1);

s += "}";

return s;

}

// ======= Web handlers =======

void handleRoot() {

server.send(200, "text/html", ptr);

}

void handleData() {

String j = jsonData();

server.send(200, "application/json", j);

}

void handleNotFound() {

server.send(404, "text/plain", "Not found");

}

// ======= Setup =======

void setup() {

Serial.begin(115200);

delay(100);

// Attach servo and set initial position

myServo.attach(SERVO_PIN);

myServo.write(0);

// OLED init

Wire.begin(D2, D1); // SDA = D2, SCL = D1

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println("SSD1306 allocation failed");

// continue without OLED

} else {

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(WHITE);

display.setCursor(0, 0);

display.println("Patient Health Monitor");

display.display();

}

// Initialize PulseOximeter

if (!pox.begin()) {

Serial.println("Failed to initialize MAX30100");

// Continue — but pox.update() will do nothing

} else {

pox.setOnBeatDetectedCallback(onBeatDetected);

pox.setIRLedCurrent(MAX30100_LED_CURR_7_6MA);

Serial.println("MAX30100 initialized");

}

// WiFi

WiFi.mode(WIFI_STA);

WiFi.begin(SSID, PASSWORD);

Serial.print("Connecting to WiFi ");

Serial.print(SSID);

unsigned long start = millis();

while (WiFi.status() != WL_CONNECTED && millis() - start < 15000UL) {

delay(250);

Serial.print(".");

}

Serial.println();

if (WiFi.status() == WL_CONNECTED) {

Serial.print("Connected! IP: ");

Serial.println(WiFi.localIP());

} else {

Serial.println("WiFi connect failed - start AP mode");

WiFi.softAP("Anesthesia_Mon_AP");

Serial.print("AP IP: ");

Serial.println(WiFi.softAPIP());

}

// Web handlers

server.on("/", HTTP_GET, handleRoot);

server.on("/data", HTTP_GET, handleData);

server.onNotFound(handleNotFound);

server.begin();

Serial.println("HTTP server started");

tsLastReport = millis();

}

// ======= Main loop =======

void loop() {

// Keep webserver responsive

server.handleClient();

// Keep pulse oximeter updated as often as possible

pox.update();

// Read sensors and update globals

unsigned long now = millis();

if (now - tsLastReport >= REPORTING_PERIOD_MS) {

// BPM & SpO2 from MAX30100

currentBPM = pox.getHeartRate();

currentSpO2 = pox.getSpO2();

// Temperature from LM35

currentTemp = readLM35Celsius();

// Debug

Serial.print("BPM: "); Serial.print(currentBPM);

Serial.print("  SpO2: "); Serial.print(currentSpO2);

Serial.print("  Temp: "); Serial.println(currentTemp);

// Update OLED if present

if (display.width() > 0) {

display.clearDisplay();

display.setCursor(0, 0);

display.setTextSize(1);

display.println("Patient Health Monitor");

display.println();

display.setTextSize(2);

display.print("HR: "); display.print(currentBPM, 0); display.println(" bpm");

display.print("SpO2: "); display.print(currentSpO2, 0); display.println("%");

display.print("Temp: "); display.print(currentTemp, 1); display.println(" C");

display.display();

}

tsLastReport = now;

}

// Threshold logic & non-blocking 10s wait

// Trigger if sensor values exceed thresholds and servo hasn't moved

if (!hasTurned && !waiting) {

// Use any condition you want; here we trigger when BPM > BPM_THRESHOLD OR Temp > TEMP_THRESHOLD

if (currentBPM > BPM_THRESHOLD || currentTemp > TEMP_THRESHOLD) {

waiting = true;

waitStart = millis();

Serial.println("Threshold exceeded — starting 10s wait (non-blocking)...");

}

}

// If waiting, check elapsed time; continue updating pox during this time

if (waiting && !hasTurned) {

if (millis() - waitStart >= WAIT_DURATION) {

// After 10s elapse, move servo once

// Optional: verify values still exceed thresholds before moving

if (currentBPM > BPM_THRESHOLD || currentTemp > TEMP_THRESHOLD) {

Serial.println("10s elapsed and threshold still exceeded — moving servo to 180°");

myServo.write(180);

hasTurned = true;

} else {

Serial.println("10s elapsed but threshold no longer exceeded — canceling");

}

waiting = false;

}

}

// small yield to allow wifi stack and background tasks

delay(20);

}